A Report On

Development of an Indian Plastic Credit Framework for Plastic Recycling

Prepared For

NOVUSCOM NEO Pvt. Ltd.

Conducted & Prepared By

INDIAN INSTITUTE OF TECHNOLOGY DELHI HAUZ

KHAS, NEW DELHI-110016

December 2025

Principal investigators: Prof Sovik Das (Civil Engineering, IIT Delhi), Prof A K Nema (Civil Engineering, IIT Delhi), Prof Sudip K. Pattanayek (Chemical Engineering, IIT Delhi)

Executive Summary

India generates approximately 9.3 million tons of plastic waste annually [1]. Of this total, polyethylene terephthalate (PET) accounts for approximately 1.28 million tons of PET waste generated annually which is expected to increase to 2.23 million tons by 2034 [2]. The PET represents a significant and economically important fraction of India's plastic waste stream due to its high recyclability (nearly 95%) and established end-use markets [3]. Despite this relatively high recyclability, substantial quantities of PET continue to leak into rivers, drains, coastal zones, floodplains, and ecologically sensitive areas. Existing plastic waste management and compliance mechanisms, including those under the Plastic Waste Management Rules and the Extended Producer Responsibility frameworks, primarily emphasise quantity-based recovery targets, with limited differentiation based on the environmental risks associated with collection locations, the quality and durability of recycling pathways, or the long-term environmental benefits of recycled products.

India’s plastic pollution challenge arises not only from plastic production but primarily from inadequate collection, spatial leakage, and limited incentives for environmentally meaningful recycling pathways. Existing regulatory mechanisms, including Extended Producer Responsibility, are largely quantity driven and do not sufficiently reward recovery from high-risk locations or the conversion of plastic into long-life recycled or upcycled products. A plastic credit system addresses this gap by shifting the focus from compliance to verified environmental service delivery, thereby recognising PET processing industries as active cleaning units rather than pollution-generating entities. By assigning value to certified collection, recycling, and upcycling outcomes, plastic credits mobilise private investment toward difficult but environmentally critical activities such as recovery from rivers, coastal zones, and ecologically sensitive areas, while promoting higher-quality recycling that extends material service life. At the same time, plastic credit systems enhance transparency, traceability, and accountability through verification and auditing, complement formal regulations, and enable structured integration of the informal sector, making them a necessary and context-appropriate instrument for advancing circular plastic management in India.

This report presents a scientifically grounded, India-specific PET Plastic Credit Framework designed to address these gaps. The framework introduces two distinct and measurable credit instruments: PET Collection Credits (PCCs) and PET Processing Credits (PPCs). Collection credits reward the verified recovery of PET waste from the environment or waste streams. In contrast, processing credits reward the conversion of collected PET into recycled or upcycled intermediate feedstock, which is intended for final product manufacturing. Credits are issued on a per-tonne basis and adjusted using location-based, ecological sensitivity-based, and product-service-life-based weightage factors. The framework is aligned with Indian regulatory and spatial planning instruments, including the Coastal Regulation Zone (CRZ) Notification, River Regulation Zone (RRZ) guidelines, and Eco-Sensitive Zone (ESZ) guidelines.

A key innovation of the framework is its spatially explicit treatment of environmental risk. The PET collection locations are categorised into land-recovered, floating, ocean-bound, and eco-sensitive zone collections, using a harmonised 500 m distance criterion for coastal, riverine, flood-prone, and ecologically sensitive landscapes. This standardised buffer captures zones of highest plastic leakage risk while maintaining methodological simplicity and consistency. Where collection occurs in overlapping sensitive zones, the framework applies the maximum applicable weightage, ensuring that compounded environmental risks are adequately reflected. This approach moves beyond broad global concepts, such as "ocean-bound plastic," and enables a more precise valuation of environmental benefits in the Indian context.

The framework further advances plastic crediting by linking processing credits to verified extension of product service life, rather than treating all recycling outcomes as environmentally equivalent. The PPCs are calculated using a life-extension function with clearly defined minimum and maximum caps, preventing over-crediting of short-lived products or unrealistically long claimed lifetimes. An additional environmental exposure factor is applied to account for products that remain directly exposed to the environment, such as geotextiles and road construction applications, thereby reflecting residual environmental risk. Together, these measures ensure that credits reward both immediate leakage prevention and long-term material retention, consistent with circular economy principles.

Strong emphasis is placed on additionality, traceability, and credibility. Credits are issued only against verifiable mass balances supported by documentation, third-party auditing, and registry-based issuance and retirement. Unique batch identification and centralised tracking mechanisms are proposed to prevent double counting. The framework is designed to be compatible with India's informal waste collection sector, while mandating compliance with labour laws, occupational health and safety standards, and social safeguards. Notably, the present framework focuses on methodological design and credit calculation principles; pricing, monetisation, and market mechanisms are intentionally excluded and identified as areas for future stakeholder consultation.

# Introduction

Plastic pollution has emerged as a significant environmental challenge in India, impacting urban environments, inland water bodies, coastal ecosystems, and ecologically sensitive areas. Despite regulatory interventions under the Plastic Waste Management Rules, 2016 and their subsequent amendments, plastic leakage into the environment persists, particularly through rivers, drains, coastal zones, and informal dumping sites. The PET constitutes a significant fraction of consumer packaging waste and is among the most widely recycled plastic polymers (nearly 95%) in the country [3]. However, existing regulatory and market-based instruments primarily emphasise quantity-based compliance, with limited consideration of environmental risk reduction, spatial sensitivity of waste leakage, or the longevity of recycled products. Consequently, there is a growing need for a structured, transparent, and scientifically defensible mechanism that explicitly rewards environmentally beneficial plastic recovery and processing.

Plastic credit systems, increasingly adopted at the global level, provide a market-based approach to mobilise private finance for plastic waste management while enhancing traceability and accountability [4]. These systems typically operate by assigning verified credits to measurable outcomes of waste collection and recycling. However, direct replication of global plastic credit models is not suitable for the Indian context due to differences in regulatory frameworks, geographic diversity, waste management infrastructure, and the predominant role of the informal sector. This underscores the need for a plastic credit framework that is specifically tailored to India's environmental, institutional, and socio-economic conditions.

A plastic credit is defined as a verified unit representing the collection/recycling of a specified quantity of plastic waste [5]. Globally, plastic credit systems distinguish between Waste Collection Credits and Waste Recycling Credits, acknowledging that collection and processing activities generate differing levels of environmental benefit and require varying degrees of effort, cost, and technological intervention [4]. Such differentiation is essential for ensuring that credits reflect real and additional environmental outcomes.

In the Indian context, plastic credits can serve as a complementary instrument to Extended Producer Responsibility by directing financial resources toward recovery from high-risk and low-access locations, enhancing the quality and traceability of recycling, and incentivising the use of long-life and environmentally preferable materials [6]. Plastic credits are particularly relevant for mitigating plastic leakage into rivers and coastal ecosystems, where even relatively small quantities of unmanaged plastic can result in disproportionately high environmental damage [7]. Accordingly, the development of a scientifically grounded, context-specific plastic credit framework represents a critical step toward strengthening India's plastic waste management and circular economy initiatives.

# Scope and Objectives

## Scope

The scope of the present framework is limited to PET plastics and is developed specifically for the Indian context. The PET has been selected due to its significant contribution to India's plastic waste stream, relatively high collection and recycling rates, and the presence of established downstream markets for recycled and upcycled PET products. The framework is applicable within India and is designed to reflect national regulatory conditions, geographic diversity, and environmental sensitivities. However, the underlying principles and methodological structure are sufficiently generic to allow adaptation by other countries, subject to contextual modifications in regulatory frameworks, geographic conditions, and waste management practices.

The proposed plastic credit system is limited to PET waste collectors and PET processors who convert collected PET into intermediate feedstock or materials intended for the manufacture of final, recycled, or upcycled products. Activities beyond feedstock preparation, such as final product manufacturing and retail, are outside the scope of the current framework. The present version of the framework focuses on methodological development and credit calculation principles. The cost, pricing, or monetisation of plastic credits is not covered and is intended to be addressed in future iterations through consultations with industry stakeholders and regulatory authorities.

## Objectives

The primary objective of this framework is to develop a scientifically sound, transparent, and verifiable plastic credit framework for PET waste management in India. To achieve this, the framework pursues the following specific objectives:

- To develop clearly defined PET plastic credits by distinguishing between collection-stage and processing-stage environmental benefits, thereby ensuring appropriate valuation of different waste management activities.

- To incorporate weightage factors based on the place of collection, with higher weightage assigned to PET recovered from environmentally sensitive or logistically challenging areas such as riverine, coastal, and eco-sensitive zones.

- To incorporate weightage factors based on the nature of the feedstock prepared for recycled or upcycled products, with emphasis on material pathways that enable longer service life and higher environmental benefit.

- To establish a transparent credit calculation methodology based on measurable quantities of PET waste, supported by documentation, traceability, and third-party verification.

# Definitions and assumptions

This section presents the key definitions, regulatory considerations, and methodological assumptions adopted in the development of the PET plastic credit framework. Emphasis is placed on spatial zoning, distance thresholds, and weightage assignment, as these directly influence credit valuation.

## Definitions

Plastic Credit: A plastic credit is defined as a verified unit representing the collection and or processing of a specified quantity of plastic waste, expressed in metric tonnes. Within the current framework, plastic credits are limited to PET plastics and are issued only after verification through documentation and third-party auditing.

PET Credit: One PET credit is defined as the verified recycling or upcycling of one metric tonne of PET plastic. PET credits are generated through two distinct activities, namely PET collection and PET processing, each of which captures different environmental benefits.

PET Collection Credit (PCC): A PET Collection Credit represents the verified recovery of PET waste from the environment or waste streams. Collection credits are calculated based on the mass of PET collected and are adjusted using location-based and ecological sensitivity-based weightage factors to reflect differences in environmental risk and recovery difficulty.

PET Processing Credit (PPC): A PET Processing Credit represents the verified conversion of collected PET into recycled or upcycled intermediate feedstock intended for final product manufacturing. Processing credits account for material losses during processing and are weighted based on the extension of service life achieved through the final product developed from processed PET.

## Collection Categories

Land-Recovered PET: It refers to PET waste collected from terrestrial locations situated beyond 500 m from the High Tide Line along the coastline and beyond 500 m from riverbanks or the highest flood levels of inland water bodies. PET in this category is not directly exposed to immediate riverine or marine transport pathways and therefore represents the lowest short-term leakage risk among the defined collection categories.

Ocean-bound PET: It refers to PET waste collected from within 500 m landward of the shoreline along the seafront, corresponding to the intertidal and landward zone of the Coastal Regulation Zone I B category. PET in this category poses a high probability of entering marine ecosystems if not recovered.

Floating PET: It refers to PET waste collected directly from rivers, canals, lakes, coastal waters, or from flood-prone riverbanks extending up to 500 m from the highest flood level recorded in the past approximately 50 years. This category represents the highest leakage risk due to direct hydrological connectivity.

Eco-Sensitive Zone PET: This refers to PET waste collected from terrestrial or aquatic locations within a 500 m radius of the notified boundary of an Eco-Sensitive Zone (ESZ), including protected forests, national parks, and wildlife sanctuaries. PET collected within this buffer is considered to pose elevated ecological risk due to proximity to sensitive habitats and biodiversity-rich areas. Accordingly, ESZ-PET is assigned an additional ecological sensitivity weightage in the plastic credit calculation framework.

## Regulatory Zonal Frameworks Considered

The development of location-based weightage factors was informed by a comprehensive review of existing regulatory zoning frameworks in India. These include the CRZ Notification issued by the Ministry of Environment, Forests, and Climate Change, the RRZ guidelines issued by the Ministry of Housing and Urban Affairs, and the ESZ notifications surrounding protected areas.

Coastal Regulation Zone: The CRZ is notified under the Coastal Regulation Zone Notification issued by the Ministry of Environment, Forest and Climate Change, Government of India [8]. The CRZ areas extend from the Low Tide Line to specified landward distances from the High Tide Line, depending on the category. For this framework, PET collected within 500 m landward of the High Tide Line along the seafront is treated as coastal-zone collection and assigned higher location-based weightage due to elevated ecological vulnerability and direct marine connectivity. Table 1 presents the CRZ categories and their notified landward and seaward extents.

Table 1 Coastal regulation zone categories and their notified landward and seaward extents

| Category | Distance Covered / Applicable Limit |
| --- | --- |
| Overall CRZ applicability | Land area from HTL up to 500 m on the landward side along the sea front is notified as CRZ. |
| CRZ-I A | Ecologically sensitive areas. No fixed horizontal distance. Includes mangroves with a 50 m buffer where mangrove area exceeds 1000 m². |
| CRZ-I B | Intertidal zone, that is, the area between HTL and LTL. |
| CRZ-II | No fixed distance limit. Applies to developed areas within the CRZ (that is, within the 500 m belt). Construction is allowed only on the landward side of existing roads or authorised structures. |
| CRZ-III A | NDZ: 50 m from HTL on landward side after approval of CZMP. Till CZMP approval: NDZ = 200 m. The overall CRZ extent still lies within 500 m of the HTL. |
| CRZ-III B | NDZ: 200 m from HTL on landward side. The remaining area, up to 500 m from the HTL, is regulated (non-NDZ). |
| CRZ-IV A | Water and seabed area from LTL up to 12 nautical miles on seaward side. |
| CRZ-IV B | Water and bed area of tidal influenced water bodies from LTL on one bank to LTL on the opposite bank, up to tidal influence limit (5 ppt salinity). |

River Regulation Zone: The RRZ framework, as referenced in guidelines issued by the Ministry of Housing and Urban Affairs, classifies riverine areas based on flood risk and development control [9]. For credit calculation purposes, PET waste recovered within 500 m of the riverbank or the highest flood level is considered riverine or floating PET and assigned an enhanced weightage, reflecting the high likelihood of downstream transport into larger river systems and coastal waters. Table 2 summarises RRZ classifications and corresponding distance limits from riverbanks or highest flood levels.

Table 2 River regulation zone classifications and distance limits

| RRZ Zone or Sub-zone | What it Covers/Activity Regulation | Distance from Riverbank/Highest Flood Level (HFL) |
| --- | --- | --- |
| Prohibited Activities Zone | Flood-prone areas and immediate floodplain - no new construction or major development allowed | Up to 500 meters from the highest flood level recorded in past ~50 years |
| Restricted Activities Zone | Buffer beyond the prohibited zone - regulated, but less strictly than prohibited zone | From outer edge of Prohibited Zone up to 1 kilometer from river / HFL |
| Regulated Activities Zone | Outer buffer where development is allowed under regulation/guidelines (not freely unrestricted) | From outer edge of Restricted Zone up to 3 kilometers from river / HFL |

Eco-Sensitive Zone: The ESZs are notified around protected forests, national parks, and wildlife sanctuaries to function as buffers for conservation [10]. Table 3 outlines ESZ buffer distances prescribed around different regions.

Table 3 Regulatory Framework for Eco-Sensitive Zones and Prescribed Buffer Distances

| Parameter | Regulatory Provision for ESZ | Remarks for Plastic Credit Framework |
| --- | --- | --- |
| Applicable Areas | Protected forests, national parks, and wildlife sanctuaries | ESZs function as buffer zones around ecologically sensitive habitats |
| Minimum ESZ Extent | 1 km from the demarcated boundary of the protected area | Represents the minimum mandatory buffer prescribed nationwide |
| General ESZ Extent | Up to 10 km from the demarcated boundary | Implemented as a general guiding principle, subject to site-specific conditions |
| Extended ESZ Extent | Beyond 10 km, where notified | Applicable for areas containing ecologically important or sensitive corridors |
| Variability Across Sites | ESZ extent differs by location and notification | Necessitates a simplified and harmonised distance criterion for credit calculations |

Although statutory definitions of CRZ, RRZ, and ESZ prescribe varying spatial extents depending on location, category, and site-specific conditions, the present framework adopts a harmonised distance criterion of 500 m for all environmentally sensitive zones for plastic credit calculation. This standardised buffer is applied uniformly across coastal, riverine, and ecologically sensitive landscapes to ensure methodological consistency, transparency, and ease of implementation.

Accordingly, PET waste collected within 500 m of the High Tide Line in coastal regions, within 500 m of the riverbank or the highest flood level in riverine systems, and within 500 m of the notified boundary of the ESZ is classified as environmentally sensitive collection. This distance represents a conservative approximation that captures zones of highest plastic leakage risk while avoiding excessive complexity arising from multiple regulatory thresholds.

Treatment of Overlapping Zones: Where PET collection locations fall within overlapping regulatory or environmentally sensitive zones, including intersections among CRZ, RRZ, and ESZ classifications, the maximum applicable location-based and ecological sensitivity weightage factor is applied. This approach ensures that compounded environmental risks are adequately reflected in credit valuation and aligns with the precautionary principle in environmental assessment.

# ESG compliance requirement and multiplier factor

While the proposed PET Plastic Credit Framework quantifies environmental benefits through risk-sensitive collection and service-life-based processing valuation, the credibility and long-term integrity of the system also depend on the governance quality, labour practices, and regulatory compliance of participating entities. Environmental outcomes should not be achieved at the expense of social safeguards, worker welfare, statutory compliance, or transparent corporate conduct. Accordingly, the framework incorporates a mandatory Environmental, Social, and Governance (ESG) compliance requirement for both PET collection and PET processing entities. This ensures that plastic credit generation is aligned with responsible business conduct and national sustainability objectives.

All entities seeking registration under the framework must possess a minimum ESG score of 50 as determined through a recognised and verifiable ESG assessment mechanism.

- Entities with ESG scores below 50 shall not be eligible to generate credits.

- The ESG assessment must be based on documented evaluation covering environmental compliance, labour and occupational safety standards, governance transparency, and statutory adherence.

- The ESG scores must remain valid at the time of credit issuance.

This minimum threshold ensures that only entities meeting baseline sustainability and compliance standards participate in the credit system.

Beyond minimum eligibility, the framework introduces a graded ESG multiplier (G) to incentivise higher standards of performance. The multiplier enhances credit issuance proportionally to the verified ESG score of the participating entity. The respective G is outlined in Table 4.

Table 4 Proposed Environmental, Social, and Governance multiplier for collection and processing units

| Tier | ESG Score Range | ESG Multiplier (G) |
| --- | --- | --- |
| Tier I | ESG Score > 80 | 1.10 |
| Tier II | ESG Score 65–80 | 1.05 |
| Tier III | ESG Score 50–65 | 1.00 |

# Plastic credits calculations

This section describes the quantitative methodology for issuing PET Collection Credits and PET Processing Credits, including the weightage factors applied, the functional form used to translate product service life into processing weightage, and worked examples that demonstrate how credits are computed in practice.

## Notation and units

- M, mass of PET involved, expressed in kilograms (kg).

- PCC = PET Collection Credit. and PPC = PET Processing Credit.

- F, location factor (dimensionless) reflecting the collection category.

- E, ecological sensitivity factor (dimensionless), reflects additional sensitivity when collection occurs within an ESZ buffer.

- T, processing life (years) that maps the service life of the finished product to a multiplier.

- P, factor to account for the environmental exposure of the processed product

- G, factor to account for the ESG compliance

## Principles

Credits are issued based on the measurable mass of PET collected and processed, adjusted by multipliers that reflect (a) the environmental risk and difficulty of collection, and (b) the environmental value of material retention measured as additional service life of the recycled or upcycled final product. Credits are issued only after documentation and third-party verification. Double-counting is prevented by unique batch identification and the registry-based retirement of credits upon issuance or sale.

## Formula for collection credits

The PCC are calculated as:

No. of PCCs = ∑Mi1000 ×Fi ×Ei ×G

Where, Mi is the mass of PET collected in kg, Fi is the location factor with values assigned according to the collection category, and Ei is an ecological sensitivity multiplier applied when collection occurs within an ESZ buffer, all from the ith location. The final no. of credits will be calculated as sum of all locations with respective location, ESG and ecological sensitivity factors. The recommended values for F and E used in this framework are provided in Table 5.

Table 5 Location and ecological sensitivity weightage factors for PET collection

| Collection category | Collection setting and spatial definition | Location factor, F | Ecological sensitivity factor, E |
| --- | --- | --- | --- |
| Land-recovered PET | PET collected from terrestrial locations beyond 500 m from coastlines and beyond 500 m from riverbanks or the highest flood levels | 1.00 | 1.00 |
| Floating PET | PET collected from rivers, canals, lakes, floodplains, and adjacent riverbanks within 500 m of the riverbank or highest flood level | 1.20 | 1.00 |
| Ocean-bound PET | PET collected from coastal land areas within 500 m landward of the High Tide Line | 1.30 | 1.00 |
| ESZ-PET | Land-recovered PET collected within 500 m of the notified boundary of an Eco-Sensitive Zone | 1.00 | 1.10 |
| ESZ-PET (floating) | Floating PET collected within 500 m of the notified boundary of an Eco-Sensitive Zone | 1.20 | 1.10 |
| ESZ-PET (ocean-bound) | Ocean-bound PET collected within 500 m of the notified boundary of an Eco-Sensitive Zone | 1.30 | 1.10 |

## Formula for processing credits

The PPC are calculated as:

No of PPCs = Mprocessed - Mwasted1000×(0.85loglog t  ×P×G

Where Mprocessed is the mass of PET fed into the processing unit in kg, Mwasted is the non-recoverable mass lost or rejected during processing in kg, t is the processing life function that maps the additional service life t, in years, achieved through the final product, and P is the weightage factor to consider the environmental exposure of the final product. For example, P is taken as one if PET is recycled back to the PET, as it prevents plastic from entering the environment. However, in the case of geotextile and road construction, the value of P is taken to be less than 1 (Table 5), as the processed product is directly exposed to the environment. Examples of some products, their lifespans, environmental exposure factors (P), and calculated weightage factors are provided in Table 6.

Table 6 Example of intermediates, products formed, their lifespan and calculated weightage factors

| PET Intermediate | Final Product | Product Category | Processing | Life (yrs) | Remarks | Notes / Example | P | Weightage (=0.85log10(t) * P) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R-PET pellets/flakes (Food grade) | Beverage bottle (e.g. water bottle) | Packaging | Recycle | 4 | Considered based on 4 no. of recycling's | Single-use (bottle-to-bottle) packaging; | 1 | 0.51 |
| R-PET nonwoven/woven fabric | Reusable shopping bag | Packaging | Downcycle | 3 | Considering life of product and no possible recycling | High-quality reusable polyester bags | 0.9 | 0.36 |
| R-PET staple fibre/yarn | Apparel (T-shirts, activewear) | Apparel | Downcycle | 3 | Considering life of product and no possible recycling | Polyester clothing typically lasts a few years | 0.9 | 0.36 |
| R-PET staple fibre (batting) | Blanket / Comforter | Home textiles (Bedding) | Downcycle | 7.5 | Considering life of product and no possible recycling | Synthetic blankets and comforters | 0.5 | 0.37 |
| R-PET staple fibre/yarn | Carpet / Rug | Home (Flooring) | Downcycle | 7.5 | Considering life of product and no possible recycling | Polyester carpets (woven or tufted pile) | 0.5 | 0.37 |
| R-PET nonwoven fabric | Geotextile (soil stabilisation) | Construction/Industrial | Downcycle | >15 | Considering life of product and no possible recycling | Geotextile fabrics buried in soil have service lives >50 years. | 0.3 | 0.30 |
| R-PET pellets (injection-moulded) | Plastic crate (stackable box) | Packaging/Logistics | Downcycle | 7.5 | Product life is higher, however directly exposed to environment | Heavy-duty plastic crates (e.g. for milk, fruits) are reused repeatedly | 0.3 | 0.22 |
| R-PET + wood flour (WPC composite) | Wall or decking panel (WPC) | Construction/Building | Downcycle | >15 | Product life is higher, however directly exposed to environment | Wood-plastic composite panels (including recycled PET/PP) | 0.3 | 0.30 |
| Shredded PET | Road construction | Construction/Building | Downcycle | 12 | Less processing required but exposed to environment | Typically, 10 to 15 years | 0.3 | 0.28 |

## Operational constraints and caps

Operational constraints and caps are introduced to ensure that processing credit remains realistic, comparable, and environmentally meaningful. These limits prevent overestimation of benefits from products with very short or excessively long claimed service lives. By applying minimum and maximum bounds, the framework maintains fairness, transparency, and credibility in credit allocation.

- The value t represents the number of extra years a product made from recycled PET can be used. For the calculation, this value must be at least 1 year. If a product does not last longer than one year, it is taken as t = 1. Since the logarithm of 1 is zero, such products do not receive any additional processing credit. In real situations, most recycled or upcycled PET products last longer than one year and therefore receive some processing credit.

- To avoid giving very high credits to products that claim extremely long lifetimes, the maximum service life considered in the calculation is limited to 15 years. If a product lasts longer than 15 years, only 15 years are used in the calculation. This means the processing credit increases with product life, but only up to a reasonable limit. At this maximum limit, the processing factor reaches a value of 1.0, ensuring that credits remain fair and balanced.

The proposed India-specific Credit Framework addresses key limitations of existing global plastic credit systems by embedding scientific differentiation, spatial risk sensitivity, and regulatory alignment directly into credit valuation. Unlike polymer-agnostic and globally uniform frameworks, this framework focuses exclusively on PET to reflect India's waste profile, recycling maturity, and market readiness, clearly separating PCC from PPC to distinguish between short-term leakage prevention and long-term material retention. By applying harmonised 500 m buffers linked to CRZ, RRZ, and ESZ regulations, it explicitly captures plastic leakage pathways and ecological vulnerability, moving beyond broad global definitions such as "ocean-bound plastic". Furthermore, the framework advances beyond volume-based crediting by linking processing credits to verified product service life and environmental exposure, with minimum and maximum caps to prevent over-crediting and assigning lower weightage to environmentally exposed applications, such as roads and geotextiles. In contrast to global systems that largely treat recycling outcomes equivalently, the proposed Indian framework ties credit value to environmental risk reduction, ecological sensitivity, and durability of material use, making it more suitable for delivering verifiable, policy-relevant environmental outcomes, as summarised in Table 7.

Table 7 Comparative Assessment of Plastic Credit Systems: India vs Global Models

| Dimension | Proposed Indian PET Plastic Credit Framework | Verra Plastic Waste Reduction Program | PCX Solutions Plastic Pollution Reduction Standard | Other Global Programs (Plastic Bank, South Pole, OBP, ZPO) |
| --- | --- | --- | --- | --- |
| Primary objective | Reduce environmental leakage of PET by rewarding risk-sensitive collection and long-life material retention | Enable global plastic collection and recycling through verified projects | Finance plastic recovery and end-of-waste processing with social safeguards | Fund plastic collection, often linked to social impact or marine litter |
| Regulatory alignment | Compliments Indian PWM Rules, CPCB EPR, CRZ, RRZ, and ESZ notifications | Not aligned to any national EPR framework | Voluntary but acknowledges EPR interaction | Generally voluntary, outside formal regulatory systems |
| Polymer coverage | PET only, selected due to high recyclability and strong Indian recycling markets | All plastic polymers and composites | All post-consumer plastics | Typically mixed plastics |
| Credit types issued | Two distinct credits: PET Collection Credits (PCC) and PET Processing Credits (PPC) | Waste Collection Credits and Waste Recycling Credits | Single plastic credit linked to end-to-end recovery | Mostly collection-based credits |
| Scientific differentiation between collection and processing | Explicit separation, reflecting different environmental benefits and effort levels | Differentiation exists but limited weighting logic | Collection and processing bundled in one credit | Often no formal differentiation |
| Spatial risk and leakage consideration | Uses harmonised 500 m buffers for coastal, riverine, and eco-sensitive zones | Project location considered but no standardised risk buffer | Uses "ocean-bound plastic" defined as within 50 km of coast | OBP uses 50 km coastal definition only |
| Ecological sensitivity consideration | Explicit ESZ multiplier for biodiversity-rich and protected areas | Not explicitly included | Not explicitly included | Generally absent |
| Treatment of overlapping sensitive zones | Applies highest applicable weightage when CRZ, RRZ, and ESZ overlap | Not formally addressed | Not formally addressed | Not addressed |
| Processing pathway valuation | Credits increase with verified extension of service life of final products | Processing credited largely on mass basis | Processing credited on recovery outcome, not longevity | Processing quality rarely differentiated |
| Service life-based weighting | Logarithmic life-extension function with minimum and maximum caps | Not considered | Not considered | Not considered |
| Environmental exposure of final product | Penalised using exposure factor P for products like roads and geotextiles | Not considered | Not considered | Not considered |
| Caps to avoid over-crediting | Minimum 1 year and maximum 15-year service life enforced | Generally linear scaling | Generally linear scaling | Usually absent |
| Informal sector suitability | Designed to integrate informal collectors within verification safeguards | Applicable but formal documentation heavy | Explicit social safeguards for informal sector | Strong informal sector focus but weaker environmental metrics |
| Traceability and verification | Batch-level documentation, third-party audits, registry-based issuance and retirement | Registry-based with third-party verification | Registry-based with VVB audits | Varies, often project-level aggregation |
| Double counting prevention | Explicitly prohibited through unique batch IDs and registry retirement | Addressed through registry controls | Addressed through registry controls | Not always transparent |
| Environmental additionality | Credits reflect difficulty of collection, ecological risk, and material longevity | Additionality demonstrated at project level | Regional baseline-based additionality | Often limited to activity-based additionality |
| Suitability for India | High. Designed specifically for India's geography, regulations, and waste systems | Moderate. Requires adaptation | Moderate. Requires adaptation | Low to moderate |
| Policy scalability | High. Methodology can be extended to other polymers or countries | High at global scale | High at global scale | Limited methodological transferability |

# CO2 savings

The proposed PET Plastic Credit Framework can deliver additional climate co-benefits by promoting increased collection, recycling, and long-term material retention of PET. By incentivising verified recovery from leakage-prone locations and encouraging processing into recycled and upcycled products, the credit system supports material substitution pathways that are associated with measurable reductions in greenhouse gas emissions. Thus, beyond preventing environmental leakage, the framework has the potential to contribute indirectly to CO₂ mitigation through enhanced recycling performance.

The CO₂ savings are associated with different recycled r-PET applications, although the magnitude varies depending on the product system, substitution pathway, and functional unit considered. For PET bottles, reported savings are 1.7 kg CO2 per kg PET, reflecting avoided emissions when recycled PET replaces virgin PET resin in bottle-to-bottle applications [11]. In textile such as recycled fabrics, savings of 1.8 kg CO2 per kg recycled cloths are reported, indicating climate benefits when r-PET fibre substitutes conventional polyester fibre [12].

For wood–plastic composite (WPC) applications, savings of 0.25 kg CO2 per kg are indicated, demonstrating comparatively lower but still positive emission reductions depending on blend composition and application type [13]. Infrastructure applications such as road construction report savings of 3.5 tons of CO2 per kilometre of road constructed (waste plastic should be 6% to 8% of the weight of bitumen in the mix), reflecting system-level emission reductions associated with partial substitution of conventional materials [14].

In addition, product-level carbon footprint values provide a basis for estimating emission benefits in cases where direct CO2 savings are not explicitly reported. The carbon footprint per kilogram of r-PET flakes is 0.43 kg CO2e per kg [13], and the carbon footprint per kilogram of r-PET fibre is 0.96 kg CO2 per kg fibre [15]. These values can be used to estimate CO2 savings for corresponding applications by comparing recycled PET products against relevant baseline materials within defined system boundaries.

Overall, the available data demonstrate that PET recycling across packaging, textile, composite, and infrastructure applications is associated with measurable greenhouse gas emission reductions. Therefore, by structurally incentivising PET collection and recycling, the proposed credit framework may generate both plastic leakage reduction benefits and ancillary climate mitigation gains.

# Worked out example for credits calculation

Data statement: A registered company collects 250 t of PET, comprising 100 t from land-recovered locations, 50 t from riverine and flood-prone areas, 40 t from coastal zones, 30 t from land areas within ESZ, 20 t from riverine ESZ locations, and 10 t from coastal ESZ locations. Out of the total PET collected, 200 t is successfully processed into useful products, while 50 t is lost or rejected during sorting and processing and is therefore not eligible for processing credits. The processed PET is converted into 80 t of bottle-to-bottle recycled PET, 40 t into PET fibre for reusable bags, 50 t into plastic crates, and 30 t into road construction applications. Consider an ESG score of 85 for both collection and processing companies.

Credits calculation: The PCC calculations are presented in Table 7 below.

Table 7 PCC calculation for the data reported in worked out example

| Collection category | PET collected (Mi, t) | Location factor (Fi) | Ecological factor (Ei) | ESG multiplier (G) | PCC (A × B × C × D) |
| --- | --- | --- | --- | --- | --- |
|  | A | B | C | D |  |
| Land-recovered PET | 100 | 1 | 1 | 1.1 | 110 |
| Floating PET | 50 | 1.2 | 1 | 1.1 | 66 |
| Ocean-bound PET | 40 | 1.3 | 1 | 1.1 | 57.2 |
| ESZ-PET | 30 | 1 | 1.1 | 1.1 | 36.3 |
| ESZ-PET | 20 | 1.2 | 1.1 | 1.1 | 29.04 |
| ESZ-PET | 10 | 1.3 | 1.1 | 1.1 | 15.73 |
| Total PCC | - | - | - |  | 314.27 |

PET Processing Credits (PPC): The PPC calculations are presented in Table 8 below.

Table 8 PPC calculation for the data reported in worked out example

| End-use application | PET processed (ton) | Service life t (yrs) | Exposure factor (P) | Processing Weightage (W = 0.85 log10(B)×C) | ESG Multiplier (G) | PPC (A×D×E) |
| --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E |  |
| Bottle-to-bottle recycling | 80 | 4 | 1.0 | 0.85 × log₁₀(4) × 1.0 = 0.512 | 1.1 | 45.1 |
| PET fibre (reusable bags) | 40 | 3 | 0.9 | 0.85 × log₁₀(3) × 0.9 = 0.365 | 1.1 | 16.1 |
| Plastic crates | 50 | 7.5 | 0.3 | 0.85 × log₁₀(7.5) × 0.3 = 0.223 | 1.1 | 12.3 |
| Road construction | 30 | 12 | 0.3 | 0.85 × log₁₀(12) × 0.3 = 0.275 | 1.1 | 9.1 |
| Total PPC | 200 | - | - | - |  | 82.46 |

# Requirements for crediting

To ensure effective, credible, and socially responsible implementation of the proposed PET plastic credit framework, the following requirements should be adhered to.

- All collection and processing activities related to plastic credit generation must comply with prevailing labour laws. This includes ensuring payment of minimum wages, provision of social security benefits where applicable, and periodic health checkups for workers involved in waste collection, sorting, and processing.

- To be eligible for earning credits, there should not be any fine imposed on the company (both collection and processing) by the state pollution and central pollution control board in last one year.

- Structured training programs should be provided to waste collectors and aggregators to improve safe handling practices and operational efficiency. Personal protective equipment such as gloves, masks, boots, and reflective gear should be mandatorily supplied. Collectors should also be clearly informed about authorised collection zones, including environmentally sensitive areas, to avoid unsafe practices and ensure compliance with regulatory boundaries adopted in the framework.

- The PET processing facilities participating in the credit system should adopt measures to prevent secondary environmental impacts. The processors should install appropriate fugitive emission control and air collection systems to minimise the release of microplastics and fine polymer particles into the surrounding environment. Compliance with air quality and workplace safety standards should be treated as a prerequisite for credit eligibility.

- Robust verification and auditing mechanisms are critical for maintaining the credibility of plastic credits. Government institutions or government-regulated and accredited third-party auditors should carry out auditing. Verification protocols should encompass mass balance, the geographic origin of collected PET, processing losses, and documentation of end-use applications to prevent double-counting and overclaiming.

- A centralised online portal should be developed to manage registration, issuance, trading, and retirement of plastic credits. The portal should enable transparent tracking of credit generation and ownership, integration with existing regulatory platforms such as the Extended Producer Responsibility system, and public visibility of retired credits to build trust among stakeholders.

Proposed work for Phase II of Plastic credit system

The present framework establishes a scientifically robust methodology for PCC and PPC. The next phase will focus on operationalising the market, strengthening additionality safeguards, building institutional capacity, and promoting innovation. The following objectives are proposed to be completed in next phase.

Credit Pricing and Market Mechanism

Phase II will develop a transparent and risk-sensitive pricing structure for PET credits. Pricing will be based on:

- Collection difficulty and environmental risk (land, floating, ocean-bound, ESZ)

- Processing complexity and material loss

- Service-life extension weightage

- ESG compliance cost

- Verification and registry administration cost

A differentiated pricing model may be adopted, where higher-risk recovery (e.g., floating or ocean-bound PET) commands premium valuation. Floor price benchmarks will be established through cost assessments and stakeholder consultation. Market mechanisms may include bilateral contracts, exchange-based trading through a central registry, or integration with EPR compliance obligations. Safeguards such as transparent retirement records and credit validity limits will prevent speculation and double counting.

Training and Capacity Building for Crediting

To ensure credible implementation, structured training programmes will be organized for:

- Collection entities: geo-tagging, zonal classification (CRZ/RRZ/ESZ), safe handling, documentation, ESG compliance.

- Processing units: mass balance accounting, service-life verification, exposure factor classification, emission control.

- Auditors and verifiers: standardised verification protocols, geographic validation, fraud prevention.

A Plastic Credit Implementation Manual will be developed, supported by periodic workshops and digital certification modules.

Strengthening Environmental Additionality

Phase II will formalise additionality assessment to ensure credits represent real and incremental environmental benefits.

Key measures will include:

- Establishing district/state-level PET leakage baselines.

- Ensuring credits exceed mandatory EPR or statutory obligations.

- Financial additionality assessment for activities dependent on credit revenue.

- Periodic baseline revision to avoid over-crediting.

- Safeguards against geographic displacement (leakage).

This structured approach will enhance credibility and long-term environmental integrity.

Introduction of R&D-Linked Credits

To promote technological advancement, a capped R&D-linked credit mechanism may be introduced for:

- Advanced sorting and traceability systems

- Microplastic emission mitigation technologies

- High-efficiency bottle-to-bottle recycling

- Innovative riverine and coastal recovery technologies

R&D credits will be milestone-based, subject to third-party validation, and capped as a proportion of total credits issued. This will incentivise innovation without diluting environmental rigor.

Implementation Roadmap

Phase II will proceed through stakeholder consultation, pricing study, registry development, pilot implementation, third-party validation, and phased national scaling.

This next phase will transition the PET Plastic Credit Framework from methodological design to a fully operational, transparent, and innovation-driven environmental market instrument capable of delivering measurable plastic leakage reduction and circular economy benefits in India.

References

[1] A. Almack, Is India the world’s largest plastic polluter? Causes and solutions, 2024. https://www.plasticsforchange.org/blog/india-emerges-as-the-worlds-largest-plastic-polluter-what-went-wrong-and-whats-next.

[2] [dataset] ChemAnalyst, India Polyethylene Terephthalate (PET) market size, growth, demand and forecast, https://www.chemanalyst.com/industry-report/india-polyethylene-terephthalate-pet-market-70.

[3] T. Team, New rule could keep 3 billion bottles out of landfills every year — India’s big move on plastic waste. https://thebetterindia.com/444327/india-pet-bottle-recycling-rule-2025/.

[4] VERRA, PLASTIC WASTE REDUCTION STANDARD. https://verra.org/programs/plastic-waste-reduction-standard/.

[5] PCx, Plastic pollution reduction standard. https://www.pcxsolutions.org/pprs.

[6] CPCB. https://cpcb.nic.in/rules-4/.

[7] Ocean Bound Plastic certification, 2016. https://www.obpcert.org/.

[8] COASTAL REGULATION ZONE NOTIFICATION, 2019, 2019. https://crz.elaw.in/crz2019.html.

[9] M.O.H.A.U.A.G.O. INDIA, RIVER CENTRIC URBAN PLANNING GUIDELINES, 2021. https://www.mohua.gov.in/upload/uploadfiles/files/RCUP.

[10] S.J. Khanduri S. K., Eco-sensitive Zones around WildlifeProtected Areas - An Evaluatory Discussion on the Public Policy,  (2025).

[11] Alpha, Study confirms the excellent carbon footprint of recycled PET, 2017. https://blog.alpla.com/en/press-release/newsroom/study-confirms-excellent-carbon-footprint-recycled-pet/08-17.

[12] TEC-Recyc., Comparison of CO2eq emissions from virgin polyester and recycled polyester - TecRecyc. https://tec-recyc.com/en/evidence.html.

[13] A. Greiner, External study confirms an excellent CO2e footprint for Greiner Packaging’s r-PET flakes, 2024. https://www.greiner-gpi.com/en_US/Newsroom/Customer-Success-Stories/External-study-confirms-an-excellent-CO2e-footprint-for-Greiner-Packaging-s-r-PET-flakes_s_354737.

[14] N.S. K., Use of Waste Plastic in Road Construction and Its Future Impacts. https://www.npcindia.gov.in/NPC/Uploads/file%20upload/6.%20Mr.%20S.K.%20Nirmal,%20Indian%20Road%20Congress.pdf.

[15] R. Zhang, X. Ma, X. Shen, Y. Zhai, T. Zhang, C. Ji, J. Hong, PET bottles recycling in China: An LCA coupled with LCC case study of blanket production made of waste PET bottles, J. Environ. Manage. 260 (2020) 110062. https://doi.org/10.1016/j.jenvman.2019.110062.